Interconnection Guide for PIMS and vetXpert Public Cloud

- This Guide Covers Only PIMS-Active Animal Sample Data Push to vetXpert

- System Connection Diagram

- System Interaction Diagram

- PIMS Settings

Notes: If the Model of operation selected “Server”, please set the IP address to IP address of the local computer where the DataSwitch software is installed.

- Instrument Setup

- Protocol Message Definition

ORR^O02 message：Complete Animal Sample Information。

ORR^O02		 General Order Response Message                   Description

MSH	Message Header

MSA	Message Acknowledgment

[PID	Animal Information

[PV1]]	Veterinary Medical Records

{

[

OBR	 Sample Information

{[OBX]}	Additional Sample Metadata Fields. Including the serial number of the instrument used, etc.

]

}

Table 1   MSH Field Definition Table

| Index | Field Name | Data Type | Maximum Recommended Length | Description | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | Field Delimiter | ST | 1 | The first field delimiter after the message segment name, which defines the delimiter value for the remaining part of the message. | ¦ |
| 2 | Encoding Characters | ST | 4 | Includes component separator, repetition separator, escape separator, and subcomponent separator. | ^~\& |
| 3 | Sending application | EI | 180 | Sending Application | Zilong |
| 4 | Sending Facility | EI | 180 | Sending Device | PIMS |
| 7 | Date/Time Of Message | TS | 26 | Message Creation Time (format: YYYY[MM[DD[HH[MM[SS]]]]]), derived from system time. | 20101012092538 |
| 9 | Message Type | CM | 7 | Message Type (format: MessageType^EventType) | ORR^O02 |
| 10 | Message Control ID | ST | 20 | Message Control ID (unique identifier for a message) | 1 |
| 11 | Processing ID | PT | 3 | Message Processing ID (values: P-Sample/worksheet query information; Q-Quality control counting result; T-System time query message; A-Actively pushed message; In Ack messages, it matches the received message.) | A |
| 12 | Version ID | VID | 60 | HL7 Version (value: '2.3.1') | 2.3.1 |
| 18 | Character Set | ID | 10 | Character Set (value: 'UNICODE' – messages are UTF-8 encoded) | UNICODE |

Table 2   MSA Field Definition Table

| Index | Field Name | Data Type | Maximum Recommended Length | Description | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | Acknowledgment Code | ID | 2 | Acknowledgment Codes： AA (Application Accept)， AE (Application Error)， AR (Application Reject)， AS (Application Skip) | AA |
| 2 | Message Control ID | ST | 20 | Message Control ID, identical to MSH-10 in the corresponding received message. | 1 |

Table 3   PID Field Definition Table

| Index | Field Name | Data Type | Maximum Recommended Length | Description | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | Set ID - PID | SI | 4 | Sequence number, used to identify different PID segments within a single message. | 1 |
| 3 | Patient Identifier List | CX | 26 | In sample messages, serves as the medical record number, formatted as 'MRN^^^^MR'. | C1^^^^MR |
| 5 | Patient Name | XPN | 48 | Name (divided into pet name, owner name, and owner telephone number), formatted as AnimalName^OwnerName^Telephone. | Wang Cai^Zhang Sheng^10086 |
| 8 | Sex | IS | 1 | Gender, string type. Matches the display string on the user interface. | Male |

Table 4   OBX Field Definition Table

| Index | Field Name | Data Type | Maximum Recommended Length | Description | Example |
| --- | --- | --- | --- | --- | --- |
| 1 | Set ID - OBX | SI | 10 | Sequence number, used to identify different OBX segments within a message. | 8 |
| 2 | Value Type | ID | 3 | Test result data type, with possible values including 'ST', 'NM', 'ED', 'IS', etc. Refer to Appendix B for details. | NM |
| 3 | Observation Identifier | CE | 590 | Data item name identifier. | 30525-0^Age^LN |
| 5 | Observation Value | * | 65535 | Data item value, which can be a number, string, enumerated value, binary data, etc. For specific value formats, refer to Appendix C. | 15 |
| 6 | Units | CE | 60 | Units. Uses HL7-defined standard unit notations. Refer to Appendix C for all units used in communication. | yr |
| 11 | Observ Result Status | ID | 1 | Value is 'F'. | F |

Table 5   Data Item Definition Table

| Data Item | Data Type (OBX-2) | Encoding (ID) | Name | Encoding Systems（EncodeSys） | OBX-3 Field Examples |
| --- | --- | --- | --- | --- | --- |
| Others Data Item |  |  |  |  |  |
| Age | NM | 30525-0 | Age | LN | # 30525-0^Age^LN |
| Age Groups | IS | 30525-1 | Age Group | LN | # 30525-1^Age Group^LN |
| Remarks | ST | 01001 | Remark | 99MRC | 01001^Remark^99MRC |
| Animal Species | IS | 01003 | Species | 99MRC | 01003^Species^99MRC |
| Instrument Serial Number | ST | 09003 | SN | 99MRC | 09003^ SN ^99MRC |

Table 6  Coded Value Table

| Data Item | Enumeration Values |
| --- | --- |
| Animal Species (Species) | Value enumeration: “720” – Dog “721” - Cat “722” - Horse “723” - Rat “724” - Mouse “725” - Rabbit “726” - Monkey “727” - Pig “728” - Camel “729” - Goat “730” - Ferret “731” - Cow “732” - Sheep “733” - Llama “734” – Panda “735” – Red Panda “736” – Guinea pig “737” – Alpaca “738” – Dolphin “739” – Whale “740” – Avian “741” – Fish “742” – Reptile “743” – Amphibian “744” – Deer “745” – Elephant “746” – Gerbil “747” – Hamster “748” – Minipig “749” – Seaturtle “750” – Tortoise “751” – Lizard “752” – Snake “753” – Cattle |
| Age Groups（Age Group） | Valid values: "Child" → Childhood "Adult" → Adulthood "Old" → Elderly "Unknown" → Unknown (default if not specified) |
| Gender | Valid values： “M”/“m”: Male “F”/“f”: Female “U”/“u”: unknow “N”/“n”:Neutered “S”/“s”:Spayed Other Content: Process directly as display strings |
| Age Units | Valid values： “Yr” -Year “mo” -Month “d” -Day “hr” -Hour “wk” -Week |

- Message Payload Demo

Animal sample information to be transmitted is as follows:

Medical Record Number：202501

Sample ID：17

Animal Type：Cat

Gender：Female

Age：6 Years Old

Age Group：Geriatric

Pet Name：Kitty

Owner Name：Trump

Phone Number：13631619220

Attending Veterinarian：Director Zhang

Laboratory Veterinarian：Dr. Li

Remarks：Test Notes

Laboratory Equipment Serial Number：SN60R-01

The corresponding transmission of animal sample reports via ORR^O02 message is as follows:

MSH|^~\&|Zilong|PMIS|||20250318172212||ORR^O02|16|A|2.3.1||||||UNICODE

MSA|AA|16||||

PID|1||202501^^^^MR||Kitty^Trump^13631619220|||Female

OBR|1||17|||||||Director Zhang||||||||||||||||||||||Dr. Li

OBX|1|NM|30525-0^Age^LN||6|yr|||||F

OBX|2|ST|01001^Remark^99MRC||Test Notes||||||F

OBX|3|IS|30525-1^Age Group^LN||Old||||||F

OBX|4|IS|01003^Species^99MRC||721||||||F

OBX|5|IS|09003^SN^99MRC||SN60R-01||||||F

The responding ACK^O02 message from vetXpert is as follows:

MSH|^~\&| | |||20230318182324||ACK^O02|16|P|2.3.1||||||UNICODE

MSA|AA|16||||0

- Special Notes

Regarding the transmission of laboratory instrument serial numbers: Since a single veterinary hospital may have multiple Mindray veterinary instruments (e.g., hematology analyzers, biochemistry analyzers), and a single animal sample may require simultaneous testing (e.g., immunology and CBC), the PIMS (Practice Information Management System) must specify which instrument(s) should be used for testing. The instrument’s serial number can be viewed in VetXpert under: [System Configuration] → [Instrument Settings].

Supported in PIMS: Specifying Multiple Devices for Testing in a Single Request. The corresponding HL7 message example is as follows:

OBX|5|IS|09003^SN^99MRC||MJ2-52001732A^MJ2-52001732A-C||||||F

Complete Communication Protocol See《HL7 Communication Protocol for vetXpert.doc》.